Uncertainties for which sampling is required (i.e., to be incorporated into Work Plan).

| Area | Uncertainty | Resolution/ Responsibility | Type of info required | Quality | Quantity | Priority |
| --- | --- | --- | --- | --- | --- | --- |
| A | What is the extent of contamination at Area A? | Excavate, use confirmation samples to delineate Plan to cleanup to Method Two | Soil samples for COC list: DRO/RRO, PCBs, VOCs |  |  |  |
|  |  | If groundwater encountered before excavation is complete | Groundwater samples for COCs that exceed Method Two levels. |  |  |  |
| B | Landfill boundaries | Survey landfill | Survey |  |  |  |
|  | Are there groundwater releases via seeps from Area B/landfill? | Visual inspection to determine possible seeps | Field visual |  |  |  |
|  |  |  | If found, sample discovered seep; likely full suite required due to landfill |  |  |  |
| C | If choose to remediate Area C, what is the extent of remediation required? | Extent of soil above Method Two and TSCA levels | Additional soil data for PCBs, DRO/RRO, VOCs either prior to or during removal If sampling conducted prior to removal, need to delineate vertically to groundwater /bedrock |  |  |  |
|  |  | Determination if groundwater is impacted at site | Visual evidence of seeps/evaluation of existing data to determine flow pathways Based on this, groundwater samples may be needed for PCBs, VOCs |  |  |  |

| Veh. Maint Yard | If relinquishing, what is the extent of soil POL contamination for UST area? | Extent of soil above Method Two | Additional soil data for DRO, RRO, GRO, BTEX, PAHs either prior to or during removal. If sampling conducted prior to removal, need to delineate vertically to groundwater /bedrock |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Determination if groundwater is impacted at site | Visual evidence of seeps/evaluation of existing data to determine flow pathways Minimum one groundwater sample at UST source location for DRO, RRO, GRO, BTEX, PAHs. Based on the sample result and groundwater understanding, additional groundwater samples may be needed for found COCs. |  |  |  |
|  | Has area west of drum storage area been impacted? | Determine presence of COCs above Method Two levels. | Soil sampling for PCBs, DRO, VOCs using multi-incremental strategy to groundwater/ bedrock; refine spacing as needed based dynamically. |  |  |  |
| WAC | Survey WAC landfill |  |  |  |  |  |
|  | If AF is planning to dispose of the land, what is extent of soil contamination? | Extent of soil above Method Two | Additional soil data for PCBs, DRO, RRO, BTEX. (PAHs need to be included w/ DRO sampling where sites are being newly characterized) PCBs to be sampled multi-incrementally If sampling conducted prior to removal, need to delineate vertically to groundwater /bedrock |  |  |  |
|  |  | Determination if groundwater is impacted at site | Visual evidence of seeps/evaluation of existing data to determine flow pathways Based on groundwater understanding, additional groundwater samples may be needed for found COCs. |  |  |  |
| LPH | Is there likelihood for contamination being left in place? | Determine if any subsurface infrastructure remains and likelihood of contamination | Subsurface survey to determine presence of graded over tanks or structures. Follow on action could include subsurface soil sampling |  |  |  |